# LoomSense Device Communication Specification v1.0

## Purpose

Define the communication contract between the LoomSense ESP32 firmware and the host application. The firmware reports hardware events only; the application interprets those events.

## Scope

The firmware monitors Hall-effect sensors, detects treadle press/release events, timestamps events, assigns a unique eventId, and transmits structured event messages. It does not manage weaving drafts, pattern position, pick tracking, user workflow, or validation logic.

## Hardware Event Message

{
  "eventId": 7,
  "timestampMs": 10624,
  "treadle": 2,
  "state": "PRESSED"
}

| Field | Type | Description |
| --- | --- | --- |
| eventId | Integer | Unique identifier assigned by the firmware for each hardware event. |
| timestampMs | Integer | Milliseconds since device startup. |
| treadle | Integer | Treadle number associated with the event. |
| state | String | Either "PRESSED" or "RELEASED". |

## Event Ordering

Events are transmitted in the order they are detected. The eventId increases by one for each event and allows the application to detect missed or duplicate messages.

## Responsibilities

Firmware:
• Monitor Hall-effect sensors
• Detect treadle state changes
• Timestamp events
• Assign eventId values
• Transmit structured event messages

Application:
• Load weaving drafts
• Track current pick and pattern position
• Validate expected treadles
• Display the user interface
• Generate alerts and statistics